Flight Dynamics: -

The area assigned to us is 0.25 ha or roughly 2500 m2. The time assigned is 10 min along with 5L

of liquid. Our UAV weighs a bit less than 14kg that fits the limit of <15 kg.

The motors used are 500 KV and Max power 500 W with 10 A. This gives us the voltage V

The max shaft power obtained is

P = V x Ah

P = 24 x 40

(Pshaft)max = 960 Wh

The thrust is given by

T=(P(ρA​)1/2)2/3

To reach the ratio of thrust to weight 2, we need to set the shaft power to at least 400 W

For a 14 kg vehicle and a required thrust-to-weight ratio of 2:

Trequired,total=214=28 kgf

Required thrust per motor:

Trequired,motor=286=4.67 kgf

The thrust required from each motor during level hover is:

Thover,motor=146=2.33 kgf

By running simulations on multiple values of RPM we have observed that although 4000 RPM is the best value to maintain a good value of the ratio, but it doesn’t support the components due to high current and power usage, thus we will stick to 3500 RPM as along with obeying the limit of the value of the ratio it also is compatible with the current and voltage limit. Thus moving forward, the static thrust is given by,

T = (2ρA(Fm.Pshaft)2)1/3

Assuming our figure of merit as 0.7 we can say,
Tstatic = 26.4 kgf

The total efficiency of the drone is in the range of 58-60 %